___________________________________________________________

IBMBIOS

IBM PC/XT BIOS Emulator for the VICTOR 9000

___________________________________________________________

Version 3.8

November 24, 1987

Ralf Kurschat

Damaschkestr.  12

7140  Ludwigsburg

(W.- Germany)

(modified by Franz Hirner)

1

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

CONTENTS

1.  FUNCTION OF IBMBIOS

2.  REQUIREMENTS

3.  INSTALLATION AND USER GUIDE

3.1  Memory reservation with 'CGINIT'

3.2  Using IBMBIOS as command interpreter

3.3  Execution of a single IBM program

3.4  Resident installation of IBMBIOS

3.5  Keyboard layout in the IBM mode

4.  INTERNAL PRINCIPLES OF IBMBIOS

4.1  BIOS calls

4.2  ANSI escape sequences

4.3  Keyboard

4.4  Characterset

4.5  Video hardware

4.6  Floppy format

4.7  Hardware port access

5.  THE IO2NMI GENERATOR

6.  KNOWN PROBLEMS

2

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

1. FUNCTION OF IBMBIOS

IBMBIOS allows to run programs  on a V9000, that are developped  mainly

for IBM PC compatibles. This is achieved by simulating a IBM PC/XT  en-

vironment.

IBMBIOS is an evolution of my older emulator 'HXTBIOS'. Now the  emula-

tor even supports  graphical programs, which would  require a CGA  card

(Color Graphics Adapter) on a IBM PC.

The programm is public domain and may be copied and distributed freely.

2. REQUIREMENTS

The emulator can only work on V9000 systems that are equipped with  896

kbyte RAM. Those  who don't have  this amount of  memory can try  other

emulators:

EMIBM     (by Daniel Cerutti)

XTBIOS    (by Ralf Kurschat)

However, the above mentioned programs do not cope with programs  acces-

sing IBM video hardware directly. Also they do not support the  graphic

modes.

When IBMBIOS is called, memory  starting in the address zone at  B000:0

must not be allocated by other programs.

Additionally for the emulation of the CGA card screen image storage  of

about 40 kbyte in  size has to be reserved within the lowest 64  kbyte.

For more details refer to chapter "3.1 Memory reservation with CGINIT".

3

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

3. INSTALLATION AND USER GUIDE

3.1 Memory reservation with 'CGINIT'

IBMBIOS is able to emulate the following IBM video cards:

a.) MDA Monochrom Display Adapter

This card only knows the text mode with 80x25 characters.  When

this card is emulated, no graphical programs can be run.

b.) CGA Color Graphics Adapter

The CGA card  knows seven screen  modes with different  resolu-

tions (max. 640x200) and number  of colors (max. 4 at a  time).

All seven modes are supported by IBMBIOS. Colors can be  repre-

sented on the V9000 by different scanning patterns.

Selection of the emulated  video card is done automatically by  IBMBIOS

by  testing, whether the  40 kbyte screen  RAM in the  lowest 64 k  was

allocated previously:

40 kbyte reserved : emulated card : remark

------------------+---------------+------------------

YES        :     CGA       : graphics possible

------------------+---------------+------------------

NO         :     MDA       : text only

------------------+---------------+------------------

Reservation of  the graphics screen memory can  be done by the  program

CGINIT.COM which you will also find on the IBMBIOS disk. CGINIT  should

be called before  other memory resident  utilities (e.g. RAMDISK).  The

best way is to call CGINIT already within the AUTOEXEC.BAT. CGINIT  re-

quires no parameters and displays:

in case of success:

"CGINIT: Memory for CGA emulation successfully allocated"

when called repeatedly:

"memory for CGA emulation was already allocated"

4

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

if no 40 kbyte memory zone is available in the lower 64 k:

"Current configuration does not allow reservation of

screen memory in the lower RAM"

CGINIT.COM works like the program  GETSCRN.EXE which is distributed  by

VICTOR together with DOS 3.1.

3.2 Using IBMBIOS as command interpreter

To call IBMBIOS without all parameters is the easiest way. IBMBIOS then

works like the command interpreter  COMMAND.COM (by simply passing  all

commands to COMMAND.COM). After entry of

IBMBIOS

without parameters and options the V9000 switches to the IBM mode and a

special prompt appears:

IBM $p >

where $p is substituted by the current path name. When this prompt  ap-

pears, all commands can be entered  and batch files or programs in  the

IBM mode can be  started. Only the shown prompt  can not be changed  by

the PROMPT command in this mode.

The V9000 stays in the IBM mode until entry of the new command

.E

IBMBIOS switches back  to Victor  mode and terminates.  The old  prompt

will then reappear.

Neither the start  of IBMBIOS nore  the entry of  the new command  '.E'

should be done from within a batch file.

5

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

3.3 Execution of a single IBM program

Instead of the above described method you can tell IBMBIOS already when

starting it the name of  a program that is to be executed in IBM  mode.

IBMBIOS then switches  immediatelly back to Victor  mode after the  IBM

program has finished. Syntax:

IBMBIOS program programparameters

Example:

IBMBIOS NORTON

Description: IBMBIOS starts, switches to IBM mode, then program  NORTON

gets startet. After NORTON terminates, IBMBIOS switches to Victor  mode

and then terminates itself.

3.4 Resident installation of IBMBIOS

IBMBIOS can  be installed residently. The V9000  will then stay in  IBM

mode until you reboot the system. Syntax:

IBMBIOS /R

Currently i don't  have written  an un-install routine,  but of  course

this would be possible.

6

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

3.5 Keyboard layout in the IBM mode

In general: Characters,  numbers and most  special characters you  will

find where they are labeled on your keyboard.

To handle IBM compatible programs, you will need some additional  keys,

which are described in the following table.

IBM keyboard function  :keys on the Victor 9000

-----------------------+----------------------------------

CONTROL                :CAPS_LOCK (will below be

:           noted as 'CNTRL')

ALT                    :ALT

CAPS_LOCK              :CONT_PAUSE

SCROLL_LOCK            :SCRL

NUM_LOCK               :CALC

ESC                    :RVS_ESC

CTRL BREAK             :CNTRL RVS_ESC

DEL                    :DEL

ALT CTRL DEL           :ALT CNTRL DEL (Bootstrap)

* PRT SCR              :* (in numeric field)

INS(0)                 :OFF_UNDL_ON

HOME(7)                :CLR_HOME

CTRL HOME(7)           :CNTRL CLR_HOME

END(1)                 :SHIFT CLR_HOME

CTRL END(1)            :CNTRL SHIFT CLR_HOME

CursorDn(2)            :CursorDown

PgDn(3)                :SHIFT CursorDown

CTRL Pg Dn(3)          :CNTRL SHIFT CursorDown

CursorUp(8)            :CursorUp

Pg Up(9)               :SHIFT CursorUp

CTRL Pg Up(9)          :CNTRL SHIFT CursorUp

Cursor<--(4)           :CursorLeft  <-

CTRL Cursor<--(4)      :CNTRL CursorLeft  <-

Cursor-->(6)           :CursorRight ->

CTRL Cursor-->(6)      :CNTRL CursorRight ->

-----------------------+----------------------------------

ALT 0 .. ALT 9         :ALT 0 .. ALT 9 (above alphabetics)

-----------------------+----------------------------------

\                      :plus/minus

:or:  ALT CNTRL =

|                      :degree

:or:  ALT CNTRL 1

[                      :1/2

{                      :1/4

:or:  ALT CNTRL 9

]                      :]

}                      :[

:or:  ALT CNTRL 0

7

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

^                      :cent

:or:  ALT CNTRL 7

<                      :,

:or:  ALT CNTRL 2

>                      :.

:or:  ALT CNTRL 3

-----------------------+----------------------------------

ALT mode of  the U.S. Victor 9000 keyboard  and the IBM compatibles  is

conflicting. The emulator therefore sends in most cases the  correspon-

ding IBM code.  To reach the  Victor character code  you can press  ALT

together with CNTRL instead of only ALT.

The 10 function keys are usable in 4 modes (normal, shift control, alt)

and send the IBM codes, not the Victor codes. The keys REQ_CAN,  WORD_-

LEFT and WORD_RIGHT can  be used instead of F8,  F9 or F10, since  some

older V9000 only have 7 function keys.

Special characters, e.g. graphical, can  be entered as on  compatibles:

Keep ALT pressed while entering the character code in decimal from  the

right numeric field. When ALT is released, the typed character is sent.

-------------------------------------

Remark of the author:

I have prepared a U.S.  version of the emulator including english  mes-

sages and  U.S. keyboard layout. However, i  have had no U.S.  keyboard

available for  tests. So if  the keyboard  is not  convenient for  your

needs, study the  assembler source listing  of the keyboard  definition

part, which i have put onto  the IBMBIOS floppy, then take a  debugger,

search for the affected data area and help yourself.

Study of this listing may also  give you all details about the  current

key definition.

--------------------------------------

4. INTERNAL PRINCIPLES OF IBMBIOS

The fourth chapter is intended  to explain the internal working of  the

IBMBIOS emulator. Those who only want to use IBMBIOS need not read it.

8

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

4.1 BIOS calls

The BIOS (basic  input/output system) provides  the low level  hardware

related  input/output functions,  like  displaying  characters  on  the

screen, scanning of the keyboard or controlling the drives.

IBM-PC/XT: Here the BIOS is stored in a PROM. All functions can be  ac-

cessed via software interrupts by supplying the necessary parameters in

the processor  registers and  executing an  INT instruction.  Interrupt

vectors # 5 through 1FH are used by the IBM BIOS.

V9000: The BIOS is loaded from disk together with the DOS. Most of  the

i/o functions are not accessed by sw interrupts but by subroutine calls

(assembler instruction CALL). The necessary entry points are not stored

at fixed memory locations.  A program requiring the BIOS entry  address

first has  to execute once  the Interrupt  0DFH. With this  call it  is

possible to get or set BIOS entry addresses and to do some other things

like changing characterset or keyboard tables.

The calling convention for the BIOS routines are therefore different on

the V9000 and on the  IBM compatibles. A V9000 will hang if a IBM  pro-

gram executes INT #10 to write something onto the screen, because on  a

V9000 there is no such function behind the INT #10 vector.

IBMBIOS directs all  on the V9000  unused interrupts 5  .. 1F onto  own

procedures that  emulate many of  the IBM BIOS  functions on the  V9000

hardware. Programs calling  the BIOS in IBM  style will therefore  work

correctly in most cases.

4.2 ANSI escape sequences

Programs written for the V9000 often contain a lot of escape  sequences

which are used to manage  the screen handling. These sequences are  de-

fined according to the VT52 definitions.

On the IBM compatibles escape sequences can be treated when the console

driver ANSI.SYS is installed. However, these ANSI sequences are  diffe-

rent from those understood by the V9000, so programs sending ANSI codes

will not work corectly on the V9000.

Unfortunately  all  ANSI  sequences  start  with  the  characters  ESC[

(CHR$(27)+CHR$(91)). Exactly this string is interpreted as the  SCREEN-

-HOLD command on a V9000. All subsequent outputs to the screen will  be

stopped and the V9000 is blocked.

IBMBIOS controls  all output to the console.  If it recognizes an  ANSI

string, it does not pass it to the screen. Instead it performs the  re-

quested function.

9

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

Programs making extensive use of VT52 escape sequences should be run in

the original  Victor mode, not  in the IBM  emulation mode, since  most

VT52 commands will not be executed in the IBM mode.

Table of ANSI escape functions,  which are treated by IBMBIOS (for  de-

tails please refer to other publications):

CHR$(27) CHR$(91) A	=	cursor up

CHR$(27) CHR$(91) B	=	cursor down

CHR$(27) CHR$(91) C	=	cursor right

CHR$(27) CHR$(91) D	=	cursor left

CHR$(27) CHR$(91) H	=	set cursor position

CHR$(27) CHR$(91) f	=	set cursor position

CHR$(27) CHR$(91) n	=	report cursor position

CHR$(27) CHR$(91) s	=	save cursor position

CHR$(27) CHR$(91) u	=	restore cursor position

CHR$(27) CHR$(91) J	=	clear home

CHR$(27) CHR$(91) K	=	clear end of line

ANSI escape functions, which are not completely handled by the  current

IBMBIOS version (but may be added in future versions):

CHR$(27) CHR$(91) m	=	set attribute

CHR$(27) CHR$(91) h	=	set screen mode (LineWrapOn)

CHR$(27) CHR$(91) l	=	reset screen mode (LineWrapOff)

CHR$(27) CHR$(91) p	=	reassign key

4.3 Keyboard

The keyboard of the  compatibles has some different keys and  different

special functions  are assigned  to keys  in comparison  to the  V9000.

Identically labeled keys may produce different character codes  between

the V9000 and IBM  compatibles (example: function keys 1..10). The  IBM

ROM BIOS also  provides the  possibility to inform  about the  physical

number of  the pressed key, which is  different from the assigned  cha-

racter code. The V9000 bios only sends the character codes.

IBMBIOS inhibits the  normal V9000 keyboard  scanning and performs  all

functions itself. Therfore  it is  able to translate  all arriving  key

codes to IBM key and character codes.

The key assignments in the IBM mode can not be changed using MODCON.

10

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

4.4 Characterset

The different character sets of the systems do not seem to be a problem

in the United States. This  chapter is therefore shortened in the  U.S.

version of IBMBIOS.DOC.

In other countries  there exist more problems,  mainly because IBM  did

not use the ASCII standard, which allows special national characters in

the character codes between 0 and 127.

An IBM compatible character set IBMPC.CHR you will find on the  IBMBIOS

disk. It can be installed using MODCON.

When the CGA card  is emulated in one of the graphics modes, a  special

font with a 8x8 matrix size becomes active. This font is integrated  in

the IBMBIOS program and can not be changed by MODCON.

4.5 Video hardware

For compatibles various different video cards are available. In  compa-

rison to the V9000 the following differences are seen:

a.) display RAM is located in different address area

b.) display attributes are on different bit positions

c.) resolution is different

d.) no color on V9000

For performance reasons many programs  written for IBM compatibles  ac-

cess the display RAM directly. On the first thought the emulator  seems

to be  unable to help  in this case,  but this is  not the case.  Under

IBMBIOS these  programs can also  run on  a V9000  with its  completely

different video organisation.

IBMBIOS uses the following principle:  It reserves officially a  memory

block at  address B000:0. B000:0 is the  starting address of the  video

memory of the IBM  monochrome card MDA, the area of the CGA RAM  begins

just behind at B800:0. Then  the IBM compatible PC program is  started.

The PC program then writes display information to Bxxx:x. This will not

directly produce a visible screen image, since the V9000 has its screen

memory at a different location, but it can not lead to errors, since no

other program uses the Bxxx:x  area. A background task of IBMBIOS  then

regularly reads the contents of the Bxxx:x IBM video RAM and constructs

the equivalent V9000 screen image.

This function is the reason that  IBMBIOS can only run on systems  with

896 kbyte RAM.

IBMBIOS checks the availability of the memory at B000:0. If this fails,

11

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

IBMBIOS terminates  immediatelly. While IBMBIOS  is active, all  memory

above 704 k is practically no more usable for other programs.

The video tranformation routine is  implemented in a way that it  first

uses up all spare time that is not used by the executing user  program,

e. g. IBMBIOS is always running  when the other program is waiting  for

keyboard input. However,  when no spare time is  left by the user  pro-

gram, IBMBIOS and the other program must share the available CPU time.

The following  table gives  an overview  of the  implementation of  the

different supported IBM video modes:

IBM mode:card :IBM resolution  :presentation on the V9000

--------+-----+----------------+--------------------------------

0    : CGA :text 25x40      :text 25x40

--------+-----+----------------+--------------------------------

1    : CGA :text 25x40 color:text 25x40 without color

--------+-----+----------------+--------------------------------

2    : CGA :text 25x80      :text 25x80

--------+-----+----------------+--------------------------------

3    : CGA :text 25x80 color:text 25x80 without color

--------+-----+----------------+--------------------------------

4    : CGA :graphics 320x200:graphics 640x400

:     : with 4 colors  : colors as scanning patterns

--------+-----+----------------+--------------------------------

5    : CGA :graphics 320x200:graphics 640x400

:     : monochrome     : gray scales as scan patterns

--------+-----+----------------+--------------------------------

6    : CGA :graphics 640x200:graphics 640x400

:     : with 2 colors  : 2 'colors': green and black

--------+-----+----------------+--------------------------------

7    : MDA :text 25x80      :text 25x80

--------+-----+----------------+--------------------------------

4.6 Floppy format

Although the V9000  and the  IBM comapatibles use  5.25" floppies,  the

physical recording format is  completely different. There are  severall

methods to copy disks from IBM  to V9000 format and vice versa.  Sorry,

but an emulator like IBMBIOS cannot help to do this.

12

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

4.7 Hardware port access

Each computer has peripheral chips like interrupt controllers or ports.

On a  V9000 these chips  are mapped  into the memory  zone beginnig  at

E000:0 and  can get  accessed via  normal read  and write  instructions

(assembler: MOV, Basic: PEEK/POKE). In the compatible PCs they are  ac-

cessed via special i/o commands  (assembler: IN, OUT) and they are  not

memory mapped.

Some IBM compatible programs try directly to access these  peripherals.

With emulator software alone it is hard to cope with these accesses  on

a V9000.  But it is  posssible to  build a simple  and cheap  extension

card, which allows to detect and emulate these i/o accesses. This  card

is described in the next chapter: "5. THE IO2NMI GENERATOR".

5. THE IO2NMI GENERATOR

On IBM compatibles peripheral  chips (ports, interrupt controller,  ti-

mer, ...) can only be accessed  by special IN / OUT instructions.  They

are not mapped into the 1 MByte memory zone of the 8088 processor as in

the V9000, but in a separate 64 kbyte large i/o zone. Accesses to  this

i/o zone are indicated by special status signals on the CPU bus.  These

signals are  available on the extension bus  connector of the V9000.  A

chip and simple circuit can be build on a card that fits into an  empty

slot and that watches the CPU status signals. Whenever an IBM style i/o

access is  recognized by the circuit, it  generates a non maskable  in-

terrupt (NMI).  The NMI vector points to  a procedure of IBMBIOS  which

analyses the access and tries to substitute it by corresponding  action

on the V9000.

Such a  card, i call  it IO2NMI  generator, is  supported from  IBMBIOS

version 3.00 upwards. At the  moment not all IBM hardware accesses  are

emulated, but future  versions of  IBMBIOS will  emulate an  increasing

number of accesses.

At the moment this IO2NMI  board is optional for the IBMBIOS  emulator.

When equipped, it helps the emulator, but the emulator will try to work

also without such a board.  However, future versions may rely more  and

more on the existence of an IO2NMI board.

IBMBIOS from version 3.01  upwards checks and reports existence of  the

IO2NMI generator during initialisation.

For those user that have some knowledge in hardware, a circuit  diagram

of the IO2NMI generator follows  below. The worth of the whole  circuit

is only few cents.

13

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

---------------------------------------+-----------------------------

S36>�+5V 13�����Ŀ                 : N1,N2,N3,N4: 1/4 TTL 7400 (NAND)

�����Ĵ N4  �                 : N5,N6:     : 1/2 TTL 7420 (NAND)

�     �     �11               : R1         : 1.5 k

�     �  &  O�����������Ŀ    : T1         : npn, SI

�   12�     �            �    +---------------------------------

� ���Ĵ     �            �

� �   �������            �

� �  2�����Ŀ            �

S07>�Oĳ��Ĵ N1  �            �

�   �     �3           �

�   �  &  OĿ          �

�  1�     � �          �

� +Ĵ     � �          �          13�����Ŀ              �S15>

�   ������� �10�����Ŀ �2�����Ŀ +�Ĵ N6  �              �

�           ��Ĵ N3  � �Ĵ N5  �  12�     �     R1       �

�  4�����Ŀ    �     �8 1�     � +�Ĵ     �8 ������Ŀ � /

S12>���O��Ĵ N2  �    �  &  O��Ĵ     �   9�  &  O�Ĵ      �Ĵ/ T1

�     �6  9�     �  5�  &  O���Ĵ     �  �������� �\

�  &  O���Ĵ     � +Ĵ     �  10�     �             Ĵ

5�     �    �������  4�     � +�Ĵ     �              �

+Ĵ     �            �Ĵ     �    �������              �

�������            � �������                         �

S10>���������������������������                                 �

�

S39>��Ŀ                                                        �

�GND�                                                    �GND�

---------------------------------------------------------------------

Pin assignment on the expansion bus connector of the V9000:

S07>  SSO	   +-------------------------------------------+

S10>  IO/-M    : SSO, IO/-M, DT/-R, -NMI are the inverted  :

S12>  DT/-R    : buffered signals of the 8088 processor    :

S15>  -NMI	   +-------------------------------------------+

S36>  +5 V

S39>  Ground

---------------------------------------------------------------------

Functional description of the IO2NMI hardware:

The circuit is located on a  card that is placed into a free slot of  a

V9000. The bus cycles are watched using the status lines SSO, IO/M  and

DT/R. During the  states "READ FROM  I/O" and "WRITE  TO I/O" the  open

collector output of T1 becomes active. The falling edge at NMI provokes

a non maskable interrupt.

14

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

6. KNOWN PROBLEMS

a.) Missing color contrast: In the graphic modes of the CGA card colors

are represented as different scanning patterns. Areas of different  co-

lor can therefore be distinguished. In  the text modes of the CGA  card

the V9000 stays for reasons of the higher speed also in the text  mode.

In this version  of IBMBIOS areas of different  color may then be  dis-

played on the  V9000 with identical screen  attributes. Input masks  of

programs working with color  information may therefore sometimes  loose

quality.

b.) Calling IBMBIOS from batch files: IBMBIOS (and other emulators like

EMIBM or XTBIOS) pass commands to COMMAND.COM. For this reason  IBMBIOS

should not be called from within  a batch file to avoid recursive  cal-

ling of COMMAND.COM.  COMMAND.COM is not  prepared to handle  recursion

perfectly. If you still do it, the following errors can occur:

- commands will not be executed at all or in different order than you

have expected

- memory allocated by processes will not be freed up completely: the

the room of free memory shrinks

This restriction is only valid for calling IBMBIOS itself. In emulation

mode all other batch files may be started.

The problem does not occur,  if you install IBMBIOS residently by  cal-

ling:

IBMBIOS /R

without any further parameter.

c.) Some programs seem to use  a nonsens font. These programs make  use

of the fact, that the IBM BIOS ROM contains a font matrix image at  ad-

dress location F000:FA6E, which they access directly (example  MS-WORD,

MS-CHART). In a  V9000 at this location  there are bootstrap  routines,

which, when used as font image, deliver complete nonsens.

Severall solution for this problem  seem to be possible. A future  ver-

sion of IBMBIOS may cover this problem.

In between patch  solutions are possible (using  a debugger). Find  out

the place where the program stes up the vector F000:FA6E to access  the

font in the ROM  and change this vector. IBMBIOS has stored a CGA  font

internally and sets up a pointer (INT #1F at 0:7C) to point to the  up-

per half of the character set  (128..255). What you need, is the  lower

half of the font image which  starts 0400H bytes below the point  where

0:7C points to.

15

. . . . . . . . . . . IBMBIOS V3.03 . . . . . . . . . . . .

d.) 'Shift PrintScreen' sends the correct IBM character code and  calls

INT #5, because there are programs which expect exactly this.  However,

the current IBMBIOS version does not perform the screen hardcopy.

This function may also be added in a future version of IBMBIOS.

e.) IBMBIOS was never tested against MS-DOS 3.1, since 3.1 seems not to

support more than 4 drives. This is not sufficient more my system.  Try

on your own and tell me about your results.

16

#######################################################################################################################