LITTLE BOARD/PLUS SCSI BOOT EPROM

---------------------------------

November 1, 1985

A. Introduction

---------------

The AMPRO SCSI/PLUS bus interface represents a significant advance in

single-board-computer architecture, by providing a general purpose, high

performance, interprocessor data channel.  The most obvious use of the

SCSI/PLUS bus interface is for the addition of SCSI (SASI) hard disk

controllers and drives.  However, this is not the only use for the SCSI/PLUS

bus.

B. Boot Algorithm

-----------------

In order to provide a general purpose "hook" for a variety of SCSI/PLUS

applications, the Little Board/PLUS SCSI BOOT EPROM allows the board to "boot"

directly from SCSI, without the need for an attached floppy disk drive.  Here

is what the new SCSI BOOT EPROM does when the board is RESET:

(1) Checks for the presence of a bootable floppy.  If one is present, it

attempts to boot from the floppy.  If this fails...

(2) Reads its SCSI ID (0-8) from the ID Input Register, and either:

(3) If the board's ID is 7, performs an SCSI bus reset and then attempts

to boot from SCSI device ID 0.  If this fails, return to step (1).

- or -

(3) If the board's ID is not 7, does not perform an SCSI bus reset.

Attempts to boot from SCSI device ID 7.  If this fails, return to step (1).

This boot algorithm provides a means to easily establish a master/slave

hierarchy, where the device which is jumpered to ID 7 is the system master.

It must be responsible to provide the bootstrap software for all devices which

are jumpered to addresses other than 7.  This process is very straight

forward; the bus master simply emulates an SCSI hard disk controller as far as

the slave processor is concerned.

C. Installation of Hard Disk Autoboot

-------------------------------------

In keeping with the general "philosophy" of SCSI, the SCSI BOOT EPROM

does not make any assumptions about the type of disk controller it is booting

from, nor about any characteristics of the hard disk drive (i.e., tracks,

heads, step rate, etc.).  For this reason, automatic booting from a hard disk

can only be done when using SCSI controllers which initialize themselves

automatically on power-up.

Of the controllers supported by the AMPRO Hard Disk Software, the

following may be used for automatic hard disk booting:

Adaptec ACB-4000 and ACB-5000 series

Shugart 1610-4 controller

Xebec OWL combination drive/controller

The Xebec 1410 and 1410A, and the DTC 510A and 510B are not self-

initializing.  These controllers require the addition of installation-specific

commands in either the SCSI BOOT EPROM or the boot strap loader (contained in

the HGEN utility), in order to initialize the controller prior to drive

access.  Hooks have been provided in the SCSI BOOT EPROM code for this

purpose.

Here is how to prepare a Little Board-based system for automatic booting

from a hard disk.  Replace your old BOOT EPROM with the new SCSI BOOT EPROM,

and then perform the following steps:

(1) Hardware Setup:  Install your system's hardware as described in Chapter 2

of the AMPRO Z80 Hard Disk Software User's Manual.  In addition, for hard disk

autoboot, SCSI ID's should be set as follows:

o  Initiator ID Jumpering:  The Little Board/PLUS is the SCSI

"Initiator."  Jumper it to SCSI ID 7, as shown in Little Board/PLUS

technical manual.

o  Target ID Jumpering:  The SCSI disk controller is the SCSI "Target."

Jumper the controller to be booted from as SCSI ID 0, according to the

controller's installation manual.  Also, the drive to be booted from

must be connected as Logical Unit Number (LUN) 0.

(2) Initial Software Setup:  Install and test the standard AMPRO Z80 Hard Disk

software and hardware as described in Chapter 2 of the Hard Disk Software

User's Manual.  This includes:

o  Running HFORMAT, to format the drive.

o  Running HINIT, to initialize your system's BIOS.

o  Running SYSGEN, to write the operating system to the first hard disk

partition.  This is "F" prior to running SWAP.

o  Running SWAP, to re-assign drive letters the way that you want them.

Be sure you have assigned drive letters so that the first hard disk

partition is now drive "A."

(3) Final Software Installation:  This is the easy part.  All you have to do

now is run the AMPRO HGEN utility, and respond to the program's prompt with a

"y."  That's all there is to it!  Remove your floppy diskettes from your

system, and press RESET.  After a brief delay (5 to 10 seconds) to allow for

floppy timeout, your system should boot directly from your hard disk.