Transferring files with KERMIT

1) Connections between two computers using RS232C:

--------------------------------------------------

1 ------------ 1

2 ------------ 3

3 ------------ 2

7 ------------ 7

4 --+      +-- 4

5 --+      +-- 5

8 --+      +-- 8

6 ----+  +---- 6

20 ----+  +---- 20

2) Connections between a computer and a modem (asynchronous)

------------------------------------------------------------

1 ------------ 1

2 ------------ 2

3 ------------ 3

4 ------------ 4

5 ------------ 5

6 ------------ 6

7 ------------ 7

8 ------------ 8

11 ------------ 11

12 ------------ 12

19 ------------ 19

20 ------------ 20

3) Connections between a computer and a modem:

----------------------------------------------

1 ------------- 1

2 ------------- 2

3 ------------- 3

4 ------------- 4

5 ------------- 5

8 ------------- 8

6 ------------- 6

20 ------------- 20

Sirius Serial Interface Pin Numbers

-----------------------------------

[From Appendix E of the MS-DOS 2.1 User's  Guide, Victor/Microsoft]

Pin	Signal				Direction

-------------------------------------------------

1	FG Frame Ground 2	TD Transmitted Data		out

3	RD Received Data		in

4	RTS Request To Send		out

5	CTS Clear To Send		in

6	DSR Data Set Ready *		in

7	SG Signal Ground

8	DCD Data Carrier Detect		in

15	TC Transmitted Clock *		in

17	RC Receiver Clock *		in

20	DTR Data Terminal Ready		out

22	Ring Indicator *		in

* These pins are used for connecting modems and are not usually

required for printers.

NOTE THAT Sirius Serial Port A is located near the centre

Sirius Serial Port B is located near the side

4) Using Kermit

------------------------

4.1 - Transferring a file:

Set speed: SET BAUD n  with n = 75 .. 9600

To Transfer: SEND <filename>

4.2 - Receive a file:

For reception: REC

4.3 - Connection to a terminal:

SET LOC OFF

CON

4.4 - Examine Kermit:

STATUS

4.5 - Quit:

EXIT

-----------------------------------------------------------------