EPSON.COM   -   Setup Utility for Epson Printers

The source code for EPSON.COM was published by Doublas Ritari in

the September 1984 issue of the PC Tech Journal.  It was modified to

run on the Victor 9000 computer by Guy Gordon.

The program allows you to send any of 8 special commands to the

Epson MX-80, in order to set the printer into desired modes.  A

second option allows enter hex codes on the command line, which are

then translated into binary and sent to the printer.  This way you

can send any escape code to any printer, though it is less

convenient than the 8 special codes.

The commands are:

E - Emphasized printing (characters are thicker than usual)

D - Double-strike printing

I - Italics

W - Wide character printing (5 char/inch)

C - Compressed character printing (17.16 char/inch)

T - Top of Form (Form Feed or top of page)

R - Restes the printer to the power-on state

U - Unidirectional printing

/xx/ - Hexadecimal input mode.  Send code xx to printer.

A typical command may read "EPSON E T" to set Emphasized

printing and go to the top of the next page.  The command "EPSON"

with no parameters will set the printer for Double-strike,

Emphasized.  The command "EPSON /1B38/ disables the paper-out

sensor, since the Epson manual says to send an "escape 8" for this

function and 1B hex is escape and 38 hex is the character "8".