CHK-FLOP v1.2

Written by Franz Hirner with much appreciated help

from John Navas.

Thursday November 3, 1988 8:06 pm

Minor editing for the P.S. Technical Wordprocessor

by R.N. Folsom, 28 April 1990

Purpose:

CHK-FLOP was initially written as an aid to recovering data that had

been written to a badly out-of- alignment Victor 9000 floppy drive.  After

several attempts at manually adjusting the drive heads resulted in the

infamous "DOS Abort, Retry, Ignore" message or system lockups, I decided to

write a program that would allow the Victor disks to run continuously

regardless of disk errors. This would allow the disk read/write head(s) to be

more easily manually adjusted until the data to be recovered could be read.

Once a drive is removed from it's cage, CHK-FLOP permits proper adjustment as

required within a few minutes.

Although originally written to recover data from badly aligned  disks,

CHK-FLOP will just as easily permit aligning a disk so that it will read a

perfectly aligned or bench mark disk.

While the principles of CHK-FLOP are not difficult and could easily be

applied to any disk drive, CHK-FLOP has been hard coded for the  Victor 9000.

The code could easily be modified to run on other computers.  Also, the

program could be enhanced to write the data read to a new file automatically.

CHK-FLOP is written to read the physical sectors on the disk rather

than the logical sectors associated with DOS files.  This permits reading

sectors 0 through 1223 on side '0' (single sided drives) or 0 through 2390 on

double sided drives.  CHK-FLOP allows the user to select the beginning sector

to read, and the number of sectors to read continuously, from either single or

double sided floppies.  Selecting side '0' (computers almost always begin

counting with zero) from the menu will limit access to legal sectors on a

single sided  drive, while selecting side '1'  allows access to the full range

of sectors on a double side drive.  The beginning sector and/or number of

sectors to read are adjusted if the user enters data that would result in

trying to read a sector below '0' or above the maximum number of sectors

available on a single or double sided drive, as selected from the menu.

Floppy Disk Alignment:

CHK-FLOP has been used successfully to not only adjust a drive to read

an unknown out-of-alignment floppy, but also to adjust a drive to properly

read data written by a disk drive that is known to be in alignment.

Alignment using CHK-FLOP is a misnomer.  True disk alignment requires

an alignment disk that has been very carefully created by the O.E.M. so as to

provide both high and low signals that can be read by a scope to insure

read/write head alignment to original specifications.  CHK-FLOP merely allows

the user to adjust the read/write heads to match a given diskette.  True

alignment may or may not result depending on the alignment of the drive that

originally produced the diskette.  The important point is that the drive can

be adjusted to read the data.  If the data happened to be written from a

aligned drive, then the disk being adjusted will be very close to a true

'aligned' drive.  With little care you can match the 'alignment' of any two

floppy disks with a fairly high degree of accuracy.

Normally, commercially produced software is distributed on disks

written with fairly accurately aligned drives and may be used for  a

substitute 'alignment disk'.  Personally, I have set aside 20 disks which have

been formated on a 'very-near-new' Victor 9000 for future use as reference

disks.

Procedure For Aligning Floppy Drives:

If you have access to a true alignment setup and A/D board then stop

here. CHK-FLOP is not meant to be a substitute for true factory alignment --

only a stop gap.  The use of an alignment disk and O.E.M. software is the best

way to align your drive.  Unfortunately, not many of us have access to the

real thing and as time goes on it will be more difficult to find a repair

facility that can/will align Victor Drives for you.

If you must align your drives, then proceed as follows.

Determine the machine to be used to create a disk that will be used as

your master disk or a diskette that you know to be 'in alignment'. If you have

selected a machine to make your reference disk, format a *NEW* unused diskette

on the subject machine.  You do want a brand new disk (or one that has been

erased with a bulk eraser) to insure that there are no latent magnetic

impulses on the disk.

Place the newly formated floppy in the drive to be 'aligned.'  Select

either "0" or '1' from the menu depending on the type of drive -- single or

double sided.  (Note computers begin counting from '0'.  A single sided drive

is '0' and a double sided drive is '1').   Select sectors at the outer,

middle, and inside positions of the drive.  Use the eccentric screw located on

the rear of the carrier to 'zero' the recording head over the track.  When

aligning the bottom read/write head limit the sectors read to values between 0

and 1223 for sector.  Use values between 1224 and 2390 when adjusting the top

read/write head. Usually it is not necessary to separately adjust both heads

unless the disk has been disassembled for repair or the heads replaced. Adjust

the eccentric screw to move the head position across the complete track. Note

that error messages will be displayed when CHK-FLOP is not properly reading

the selected sector but the disk will not stop spinning.  DOS keeps track of

the sector being read and reports  an error if you go to far and position the

head over an adjacent track.   No error messages are displayed when CHK-FLOP

*IS* successfully reading the sector selected.  Carefully note the amount of

movement that you can make on the eccentric adjustment screw without moving

the head to the point that errors are produced.  Split the difference and you

should be very close to 'in-the money.'  Repeat on at least outer, middle, and

inner sectors until you are satisfied.  Difference in the stepper motor from

one drive to another may require slight fine tuning of the adjustment between

inner, middle, and outer sectors.

Usually, it is not necessary to separately adjust the top or '1'

read/write head unless the head has been replaced or removed, but you should

check this head if you have double sided drives.  If adjustment is necessary,

you must use a trial and error position since the top head is not equipped

with an eccentric adjustment.

After side '0' has been adjusted to your satisfaction lock the three

carrier retainer screws firmly. It's a good idea to format a new disk in the

drive you have just aligned and confirm that you can read all sectors on the

machine you used to create the master disk before putting the machine back

together.

CHK-FLOP, while perhaps crude by professional standards, does make it

easy to mach the alignment of one drive to another.  Don't use it  unless you

have to, but when you have to recover data from a 'out- of-alignment' disk

CHK-FLOP may save the day.

####################################################################################################################