Notes by R.N. Folsom, 26 November 1990

DSKBENCH.EXE is a public domain disk testing routine, written by John

Navas, that tests hard disk speed.

I obtained DSKBENCH.EXE from the Bavug bulletin board, (415)574-7914,

Franz Hirner, sysop.

R.N. Folsom

128 Seafoam Avenue

Monterey, California  93940

(408)649-6383 or (408)924-5418 or -5400

John Navas' Original Documentation:

The DSKBENCH disk benchmark program uses only documented DOS calls

and so will run on ANY MS-DOS microcomputer.  The current drive is

always tested and there must be at least 360 KB of space available

for the tests to be run.  Best results are obtained with an empty

or defragmented disk.

Revision history:

1.2	Ensures that the I/O buffer does not cross a 64KB memory

boundary (which can adversely affect performance).

1.3	Corrects a bug in the buffer allocation routines

introduced in Version 1.2.

1.4	Improved error-handling.

1.5	General clean-up.

1.6	Ensures that the temporary file is always located on the

default drive.

1.7	Restarts timing if a sector read error occurs instead of

aborting.

Reported results:

Victor 9000 w/Tandon 100-4 1.2MB Floppy Drive

Block  I/O    read   20.9 kb/sec.    write    8.4 kb/sec.

Sector I/O    read    2.2 kb/sec.    write    0.8 kb/sec.

DOS random sector read  470.0 ms. avg.

Effective rotation speed    252 rpm

Victor 9000 w/Tandon 10 MB Hard Drive

Block  I/O    read   76.7 kb/sec.    write   76.7 kb/sec.

Sector I/O    read   13.1 kb/sec.    write    9.2 kb/sec.

DOS random sector read  117.1 ms. avg.

Effective rotation speed   1800 rpm*

Victor 9000 w/30 MB Rodime Hard Drive

Block  I/O    read   72.1 kb/sec.    write   81.9 kb/sec.

Sector I/O    read   13.1 kb/sec.    write    9.0 kb/sec.

DOS random sector read   86.7 ms. avg.

Effective rotation speed   1800 rpm*

Victor 9000 w/Priam VT-150 40 MB. Hard Drive

Block  I/O    read   72.1 kb/sec.    write   80.1 kb/sec.

Sector I/O    read   13.4 kb/sec.    write    9.2 kb/sec.

DOS random sector read   50.0 ms. avg.

Effective rotation speed   1800 rpm*

Victor 9000 w/Atasi 40 MB Hard Drive

Block  I/O    read   76.7 kb/sec.    write   75.1 kb/sec.

Sector I/O    read   13.4 kb/sec.    write    9.4 kb/sec.

DOS random sector read   53.3 ms. avg.

Effective rotation speed   1800 rpm*

Victor 9000 w/NEC & Tandon 703 30Mb Drive:

Block  I/O    read   76.7 kb/sec.    write   90.1 kb/sec.

Sector I/O    read   14.1 kb/sec.    write    9.4 kb/sec.

DOS random sector read   56.0 ms. avg.

Effective rotation speed   1800 rpm*

* NOTE: Since these drives actually rotate at 3600 rpm, it appears

that the Victor always loses a revolution when rereading the same

sector.

Victor 9000 PLUSPC & Cogito 912 10Mb Drive:

Block  I/O    read   78.5 kb/sec.    write   74.9 kb/sec.

Sector I/O    read   23.0 kb/sec.    write    9.2 kb/sec.

DOS random sector read  200.0 ms. avg.

Effective rotation speed   3649 rpm

Victor VI (IBM Mode) w/Rodime 30 Meg. Block  I/O    read   71.4 kb/sec.    write   71.2 kb/sec.

Sector I/O    read   21.7 kb/sec.    write    9.1 kb/sec.

DOS random sector read   78.7 ms. avg.

Effective rotation speed   3556 rpm

Victor VI (Victor Mode) w/Rodime 30 Meg.

Block  I/O    read   70.7 kb/sec.    write   69.8 kb/sec.

Sector I/O    read   21.8 kb/sec.    write    9.1 kb/sec.

DOS random sector read   78.7 ms. avg.

Effective rotation speed   3546 rpm

Victor 9000 w/ White Crane RAMDISK (500K)

Block  I/O    read  270.3 kb/sec.    write  206.0 kb/sec.

Sector I/O    read   73.2 kb/sec.    write   63.2 kb/sec.

DOS random sector read    2.3 ms. avg.

Effective rotation speed  38250 rpm

Victor 9000 w/WA Escort 1.5 Board RAMDRIVE

Block  I/O    read  468.6 kb/sec.    write  439.6 kb/sec.

Sector I/O    read   74.2 kb/sec.    write   54.9 kb/sec.

DOS random sector read    2.7 ms. avg.

Effective rotation speed  27000 rpm